# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Supply of Smart system purchase with installation	p 1 / 7

Publication reference: HUSRB/1602/11/0225/Municipality of Novi Becej/TD6-2

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Novi Becej "Smart" system 1 set consisting of |  |  |  |
| 1.1 | Instruments to be installed in monitoring well 1 set: Data logger parameters Storing water level, temperature data Data transmission GSM / GPRS Frequency 900 / 1800 MHz (standard); 850 / 1900 MHz (alternative) Memory min 2 MB for up to 190.000 measuring values Interface Output to Notebook: RS 232-serial or USB; input from measuring probes: 2 x RS 485 Modbus-connections or 1 x analogue 4 to 20 mA Energy supply Standard: 3 pc. alkali-manganese battery size C or 3 pc. lithium batteries size C or better Measuring interval adjustable, 1 second to 99 days Dimensions approx Ø 49 mm x 380 mm - including battery compartment and antenna Assembly in level pipe 2“ to 6“ pipes; from 3“ with fall protection rings Material Housing in high-grade stainless steel / high-quality synthetic Type of protection IP 68, permanently flood-proof up to 1,5 m hydrostatic head Water Pressure Probe Parameters Water level adjustable; up to 300 m hydrostatic head Temperature of operation 0 to +50 °C or better Accuracy < 0,05 % of measuring range (water level) Accuracy < 0,1 °C (temperature) Dimensions approx Ø 22 mm x 120 mm Material high-grade stainless steel Communication • Wireless interchangeable SIM card service providing independent data (GPRS) General Informations Data format Readout of data in ASCII or Excel-format Change of battery possible to be done by user Cable of probes Special cable (Ø 6 mm): UV-resistant Polyurethan-manteling with Kevlar core and inside air hose for compensation of barometric fluctuations, with no cartridges necessary; Assembly device to fit into level pipes with the possibility to do back-check measurements with an electric contact gauge without the need to disassemble the probe(s) Accessories USB transmission cable 5m, user software. Supported temperature compensation from 0 to +50 °C Integration into a telemetry system o Data over a wireless network must reach the unit's remote metering center and store it there in a cloud or FTP server or computer o To receive and retrieve the data, the supplied remote metering software must be used and contractor must incorporate a device that stores the data on the site, download data from the operation of the remote measurement |  |  |  |
| 1.2 | Guided water supply to the flooded area or vice versa – Monitoring Module - 1 set Technical specification and summary of discharge measurement unit: Water flow and speed measuring system TECHNICAL DETAILS GENERAL Protection class IP 66 Battery 24 Ah / 12 V Solar panel Power consumption active operation up to 150 mA Power consumption stand by max 3 mA Operating temperature - 20° ...50° C or better Over voltage protection, reverse power protection, lightning protection WATERLEVEL MEASUREMENT Measuring range min 0 ... 15 m Resolution min 1 mm Accuracy min +/- 2 mm Radar frequency 26 GHz (K-Band) Radar opening angle 10° VELOCITY MEASUREMENT Measuring range min 0.10 ... 15 m/s Accuracy min +/- 0.01 m/s; +/- 1 % FS Resolution min 1 mm/s Direction recognition +/- Measurement duration 5 ... 240 sec. Measurment interval 8 sec. ... 5h Measuring frequency 24 GHz (K-Band) Radar opening angle 12° Distance to water surface supported 0.50 ... 35 m DATA TRANSMISSION Supported networks quad band EGSM 850 / 900 / 1800 / 1900 MHz networks 2G/3G/4G asynchronous, transparent, circuit-switched communication method up to 14.4 kbps; asynchronous, non-transparent, circuit-switched communication method up to 9.6 kbps supported Data transfer FTP server or HTTP server: CSV or xHydro format via ftp protocol; MDS server: CSV format via http protocol. DATA LOGGER Storage up to min 500,000 values Data readout remotely Single point of connection principle Remote access to the data logger and RQ supported |  |  |  |